目录

File & folder	3

Test case	3

C查阅工具	6

常用文件	6

头文件关系	8

Code analysis	8

soc_new_request	8

init_ip_request	9

dma_populate_request	10

soc_allocate	10

dma_init_context	11

create_context_dma_inst	11

create_context_dma3	12

soc_get_context_pointer	13

ip_request_t	13

dma_context_t	14

DMA3_CONTEXT	14

dma3_reg_check_reset	15

RE32	16

dma3_reg_check_complement_32bit	16

WRE32	18

Register	18

中断工作流程	18

dma3_setup_int(dma3_context);	18

dma3_setup_hw	19

Register definition	21

DMA3_TCD_SADDR	21

DMA3_TCD_SOFF	22

DMA3_TCD_ATTR	23

DMA3_TCD_NBYTES	24

DMA3_TCD_SLAST	26

DMA3_TCD_DADDR	28

DMA3_TCD_CITER	29

DMA3_TCD_DOFF	30

DMA3_TCD_DLAST	31

DMA3_TCD_BITER	32

DMA3_TCD_CSR	34

DMA3_CH_SBR	37

DMA3_CH_CSR	39

DMA3_CH_PRI	40

DMA3_GRPRI	41

Makefile	42

Testbench overview knowledge	43

Module description	44

TCD	44

Channel preemption	45

Scat/gather	46

Example	46

Channel linking	46

Example	47

# 变量定义

## CAPI 地址

/home/imxrt700_verif1/richard_nxa28190/V_SS_RT700_SOC_TB_1.16/testbench/blocks/soc_tb/testbench/instances_v/cm33_ahb_reporter_inst.sv

cm33_cprint_regs_base =  `CM33_CPRINT_REGS_BASE;

cm33_cprint_cmd_addr = cm33_cprint_regs_base + `CPRINT_CMD_OFFSET;

cm33_cprint_arg_addr = cm33_cprint_regs_base + `CPRINT_ARG_OFFSET;

cm33_cprint_rsp_addr = cm33_cprint_regs_base + `CPRINT_RSP_OFFSET;

/home/imxrt700_verif1/richard_nxa28190/V_SS_RT700_SOC_TB_1.16/testbench/blocks/soc_tb/tool_data/sray/mem_map/ capi.mem_map

`define CM33_CPRINT_REGS_BASE              'h3000FFE0

`define CPRINT_ARG_OFFSET        'h08

`define CPRINT_CMD_OFFSET        'h00

`define CPRINT_RSP_OFFSET        'h08

Testbench/comon_blocks/v_ip_capi/tool_data/compiler/include/capi_mmp.h

#define CPRINT_CMD          (*((vcommand_t*)((CPRINT_REGS_BASE)+(CPRINT_CMD_OFFSET))))

#define CPRINT_ARG          (*((vuint8_t*)((CPRINT_REGS_BASE)+(CPRINT_ARG_OFFSET))))

#define CPRINT_RSP          (*((vuint8_t*)((CPRINT_REGS_BASE)+(CPRINT_RSP_OFFSET))))

## MBOX 地址

### GPIO example

/home/imxrt700_verif1/richard_nxa28190/V_SS_RT700_SOC_TB_1.16.1.1/testbench/blocks/soc_tb/testbench/top_defines_v/mem_map.v

#define RGPIO_DRV_PIN_PIN_MBOX   (RGPIO_TBCOMM_BASE + RGPIO_DRV_PIN_PIN_TBCOMM_OFFSET  )

#### RGPIO_TBCOMM_BASE

testbench/instances_v/rgpioc_drv_inst.sv:

`define RGPIOC_DRV_WRAPPER_INST(num) \

rgpio_driver_monitor  #(.RGPIO_TBCOMM_BASE(`RGPIO``num``_TBCOMM_BASE),     \

.RGPIO_WIDTH(`RGPIO``num``_WIDTH))                 \

rgpioc``num``_drv_wrapper(                                              \

.ipg_stop                (`RGPIO``num``_STOP),                       \

.ipg_wait                (`RGPIO``num``_WAIT),                       \

.ipg_other_plat_stop     (`RGPIO``num``_STOP_other),                 \

.ipg_other_plat_wait     (`RGPIO``num``_WAIT_other),                 \

.tb_ipg_clk              (`RGPIO``num``_TB_IPG_CLK),                 \

.gpio_port               (`RGPIO``num``_PORT_IF.vip),                \

.gpio_ovr_port           (`RGPIO``num``_OVR_PORT_IF.vip)             \

);

`PP_REPEAT(`NUM_RGPIO_INSTS, RGPIOC_DRV_WRAPPER_INST)

testbench/blocks/soc_tb/tool_data/sray/mem_map

gpio_params.mem_map:

`define RGPIO0_TBCOMM_BASE      `GPIO0_TBCOMM_BASE

tbcomm.mem_map:

`define GPIO0_TBCOMM_BASE     'h0040*152

#### RGPIO_DRV_PIN_PIN_TBCOMM_OFFSET

tool_data/sray/mem_map/rgpio.mem_map:82:

`define RGPIO_DRV_PIN_PIN_TBCOMM_OFFSET             'h0

## command 定义

Testbench/comon_blocks/v_ip_capi/tool_data/compiler/include/capi_mmp.h

#define CPRINT_NOP              (0x00)

#define CPRINT_TRIGGER_SET      (0x01)

#define CPRINT_TRIGGER_GET      (0x02)

#define CPRINT_MB_PUT8          (0x03)

#define CPRINT_MB_GET8          (0x04)

#define CPRINT_MB_PUT16         (0x05)

#define CPRINT_MB_GET16         (0x06)

#define CPRINT_MB_PUT32         (0x07)

#define CPRINT_MB_GET32         (0x08)

#define CPRINT_MB_PUT64         (0x09)

#define CPRINT_MB_GET64         (0x0A)

#define CPRINT_RESERVED_0C      (0x0B)

#define CPRINT_RESERVED_0D      (0x0C)

#define CPRINT_RESERVED_0E      (0x0D)

#define CPRINT_RESERVED_0F      (0x0E)

#define CPRINT_FLAG_SET         (0x0F)

#define CPRINT_FLAG_CLEAR       (0x10)

#define CPRINT_FLAG_WAIT        (0x11)

#define CPRINT_FLAG_CHECK       (0x12)

#define CPRINT_DEBUG            (0x13)

#define CPRINT_TRACE            (0x14)

#define CPRINT_INFO             (0x15)

#define CPRINT_WARN             (0x16)

#define CPRINT_ERROR            (0x17)

#define CPRINT_FATAL            (0x18)

#define CPRINT_SHOW8            (0x19)

#define CPRINT_SHOW16           (0x1A)

#define CPRINT_SHOW32           (0x1B)

#define CPRINT_SHOW64           (0x1C)

#define CPRINT_SHOWA            (0x1D)

#define CPRINT_PRINTF           (0x1E)

#define CPRINT_RESERVED_1F      (0x1F)

#define CPRINT_RESERVED_20      (0x20)

#define CPRINT_RESERVED_21      (0x21)

#define CPRINT_R8               (0x22)

#define CPRINT_R16              (0x23)

#define CPRINT_R32              (0x24)

#define CPRINT_R64              (0x25)

#define CPRINT_INFO_RE8         (0x26)

#define CPRINT_INFO_RE16        (0x27)

#define CPRINT_INFO_RE32        (0x28)

#define CPRINT_INFO_RE64        (0x29)

#define CPRINT_INFO_REM8        (0x2A)

#define CPRINT_INFO_REM16       (0x2B)

#define CPRINT_INFO_REM32       (0x2C)

#define CPRINT_INFO_REM64       (0x2D)

#define CPRINT_INFO_RRE8        (0x2E)

#define CPRINT_INFO_RRE16       (0x2F)

#define CPRINT_INFO_RRE32       (0x30)

#define CPRINT_INFO_RRE64       (0x31)

#define CPRINT_INFO_RREM8       (0x32)

#define CPRINT_INFO_RREM16      (0x33)

#define CPRINT_INFO_RREM32      (0x34)

#define CPRINT_INFO_RREM64      (0x35)

#define CPRINT_INFO_RELU8       (0x36)

#define CPRINT_INFO_RELU16      (0x37)

#define CPRINT_INFO_RELU32      (0x38)

#define CPRINT_INFO_RELU64      (0x39)

#define CPRINT_INFO_RELS8       (0x3A)

#define CPRINT_INFO_RELS16      (0x3B)

#define CPRINT_INFO_RELS32      (0x3C)

#define CPRINT_INFO_RELS64      (0x3D)

#define CPRINT_ERROR_RE8        (0x3E)

#define CPRINT_ERROR_RE16       (0x3F)

#define CPRINT_ERROR_RE32       (0x40)

#define CPRINT_ERROR_RE64       (0x41)

#define CPRINT_ERROR_REM8       (0x42)

#define CPRINT_ERROR_REM16      (0x43)

#define CPRINT_ERROR_REM32      (0x44)

#define CPRINT_ERROR_REM64      (0x45)

#define CPRINT_ERROR_RRE8       (0x46)

#define CPRINT_ERROR_RRE16      (0x47)

#define CPRINT_ERROR_RRE32      (0x48)

#define CPRINT_ERROR_RRE64      (0x49)

#define CPRINT_ERROR_RREM8      (0x4A)

#define CPRINT_ERROR_RREM16     (0x4B)

#define CPRINT_ERROR_RREM32     (0x4C)

#define CPRINT_ERROR_RREM64     (0x4D)

#define CPRINT_ERROR_RELU8      (0x4E)

#define CPRINT_ERROR_RELU16     (0x4F)

#define CPRINT_ERROR_RELU32     (0x50)

#define CPRINT_ERROR_RELU64     (0x51)

#define CPRINT_ERROR_RELS8      (0x52)

#define CPRINT_ERROR_RELS16     (0x53)

#define CPRINT_ERROR_RELS32     (0x54)

#define CPRINT_ERROR_RELS64     (0x55)

#define CPRINT_W8               (0x56)

#define CPRINT_W16              (0x57)

#define CPRINT_W32              (0x58)

#define CPRINT_W64              (0x59)

#define CPRINT_INFO_WR8         (0x5A)

#define CPRINT_INFO_WR16        (0x5B)

#define CPRINT_INFO_WR32        (0x5C)

#define CPRINT_INFO_WR64        (0x5D)

#define CPRINT_INFO_WRM8        (0x5E)

#define CPRINT_INFO_WRM16       (0x5F)

#define CPRINT_INFO_WRM32       (0x60)

#define CPRINT_INFO_WRM64       (0x61)

#define CPRINT_INFO_WRE8        (0x62)

#define CPRINT_INFO_WRE16       (0x63)

#define CPRINT_INFO_WRE32       (0x64)

#define CPRINT_INFO_WRE64       (0x65)

#define CPRINT_INFO_WREM8       (0x66)

#define CPRINT_INFO_WREM16      (0x67)

#define CPRINT_INFO_WREM32      (0x68)

#define CPRINT_INFO_WREM64      (0x69)

#define CPRINT_RMW8             (0x6A)

#define CPRINT_RMW16            (0x6B)

#define CPRINT_RMW32            (0x6C)

#define CPRINT_RMW64            (0x6D)

#define CPRINT_INFO_RMWR8       (0x6E)

#define CPRINT_INFO_RMWR16      (0x6F)

#define CPRINT_INFO_RMWR32      (0x70)

#define CPRINT_INFO_RMWR64      (0x71)

#define CPRINT_ERROR_WR8        (0x72)

#define CPRINT_ERROR_WR16       (0x73)

#define CPRINT_ERROR_WR32       (0x74)

#define CPRINT_ERROR_WR64       (0x75)

#define CPRINT_ERROR_WRM8       (0x76)

#define CPRINT_ERROR_WRM16      (0x77)

#define CPRINT_ERROR_WRM32      (0x78)

#define CPRINT_ERROR_WRM64      (0x79)

#define CPRINT_ERROR_WRE8       (0x7A)

#define CPRINT_ERROR_WRE16      (0x7B)

#define CPRINT_ERROR_WRE32      (0x7C)

#define CPRINT_ERROR_WRE64      (0x7D)

#define CPRINT_ERROR_WREM8      (0x7E)

#define CPRINT_ERROR_WREM16     (0x7F)

#define CPRINT_ERROR_WREM32     (0x80)

#define CPRINT_ERROR_WREM64     (0x81)

#define CPRINT_ERROR_RMWR8      (0x82)

#define CPRINT_ERROR_RMWR16     (0x83)

#define CPRINT_ERROR_RMWR32     (0x84)

#define CPRINT_ERROR_RMWR64     (0x85)

#define CPRINT_INFO_WAIT_RE8    (0x86)

#define CPRINT_INFO_WAIT_RE16   (0x87)

#define CPRINT_INFO_WAIT_RE32   (0x88)

#define CPRINT_INFO_WAIT_RE64   (0x89)

#define CPRINT_INFO_WAIT_REM8   (0x8A)

#define CPRINT_INFO_WAIT_REM16  (0x8B)

#define CPRINT_INFO_WAIT_REM32  (0x8C)

#define CPRINT_INFO_WAIT_REM64  (0x8D)

#define CPRINT_ERROR_WAIT_RE8   (0x8E)

#define CPRINT_ERROR_WAIT_RE16  (0x8F)

#define CPRINT_ERROR_WAIT_RE32  (0x90)

#define CPRINT_ERROR_WAIT_RE64  (0x91)

#define CPRINT_ERROR_WAIT_REM8  (0x92)

#define CPRINT_ERROR_WAIT_REM16 (0x93)

#define CPRINT_ERROR_WAIT_REM32 (0x94)

#define CPRINT_ERROR_WAIT_REM64 (0x95)

#define CPRINT_BSET8            (0x96)

#define CPRINT_BSET16           (0x97)

#define CPRINT_BSET32           (0x98)

#define CPRINT_BSET64           (0x99)

#define CPRINT_BCLR8            (0x9A)

#define CPRINT_BCLR16           (0x9B)

#define CPRINT_BCLR32           (0x9C)

#define CPRINT_BCLR64           (0x9D)

#define CPRINT_BINV8            (0x9E)

#define CPRINT_BINV16           (0x9F)

#define CPRINT_BINV32           (0xA0)

#define CPRINT_BINV64           (0xA1)

#define CPRINT_RERR8            (0xA2)

#define CPRINT_RERR16           (0xA3)

#define CPRINT_RERR32           (0xA4)

#define CPRINT_RERR64           (0xA5)

#define CPRINT_WERR8            (0xA6)

#define CPRINT_WERR16           (0xA7)

#define CPRINT_WERR32           (0xA8)

#define CPRINT_WERR64           (0xA9)

// EXIT simulation commands

#define CPRINT_PASS             (0xAA)

#define CPRINT_FAIL             (0xFF)

// -----------------------------------------------------------------------------

// end CPRINT memory map

// -----------------------------------------------------------------------------

#endif // CAPI_MMP_H

## 变量类型宏定义

Testbench/blocks/soc_tb_tool_data/compiler/include/capi_soc.h

typedef uint8_t  command_t;

#ifndef AARCH64

typedef uint32_t address_t;

#else

typedef uint64_t address_t;

#endif

typedef uint8_t  err_cnt_t;

typedef uint16_t trigger_t;

typedef uint16_t flag_t;

typedef uint16_t mailbox_t;

typedef uint16_t tbcomm_t;

typedef uint8_t trigger_status_t;

typedef uint8_t flag_status_t;

typedef uint8_t mailbox_status_t;

# V

testbench.cm33_ahb_reporter

# C

## RE8, WR8等函数的定义在

Testbench/common_blocks/v_ip_capi/tool_data/compiler/include/capi_api.h

#define WR32(address, write_data)                               capi_wr32((vuint32_t*)&(address), (uint32_t)(write_data))

#define WRM32(address, write_data, mask)                        capi_wrm32((vuint32_t*)&(address), (uint32_t)(write_data), (uint32_t)(mask))

#define WRE32(address, write_data, expect_data)                 capi_wre32((vuint32_t*)&(address), (uint32_t)(write_data), (uint32_t)(expect_data))

#define WREM32(address, write_data, expect_data, mask)          capi_wrem32((vuint32_t*)&(address), (uint32_t)(write_data), (uint32_t)(expect_data), (uint32_t)(mask))

#define RMW32(address, write_data, mask)                        capi_rmw32((vuint32_t*)&(address), (uint32_t)(write_data), (uint32_t)(mask))

#define RMWR32(address, write_data, mask)                       capi_rmwr32((vuint32_t*)&(address), (uint32_t)(write_data), (uint32_t)(mask))

#define WAIT_RE32(address, expect_data, read_attempts)          capi_wait_re32((vuint32_t*)&(address), (uint32_t)(expect_data), (uint16_t)(read_attempts))

#define WAIT_REM32(address, expect_data, mask, read_attempts)   capi_wait_rem32((vuint32_t*)&(address), (uint32_t)(expect_data), (uint32_t)(mask), (uint16_t)(read_attempts))

#define BSET32(address, mask)                                   capi_bset32((vuint32_t*)&(address), (uint32_t)(mask))

#define BCLR32(address, mask)                                   capi_bclr32((vuint32_t*)&(address), (uint32_t)(mask))

#define BINV32(address, mask)                                   capi_binv32((vuint32_t*)&(address), (uint32_t)(mask))

#define RERR32(address)                                         capi_rerr32((vuint32_t*)&(address))

#define WERR32(address, write_data)                             capi_werr32((vuint32_t*)&(address), (uint32_t)(write_data))

## MB_PUT8

MB_PUT8 (RGPIO_DRV_PIN_PIN_MBOX,        pin);   //0

MB_PUT8 (RGPIO_DRV_PIN_VALUE_MBOX,      value);  //0

MB_PUT8 (RGPIO_DRV_PIN_WAIT_MBOX,       in_wait);  //0

MB_PUT8 (RGPIO_DRV_PIN_STOP_MBOX,       in_stop);  //0

MB_PUT8 (RGPIO_DRV_PIN_OTHER_PLAT_MBOX, other_plat); //0

MB_PUT32(RGPIO_DRV_PIN_LATENCY_MBOX,    latency); //0

MB_PUT8 (RGPIO_DRV_PIN_TOGGLE_MBOX,     toggle); //0

MB_PUT32(RGPIO_DRV_PIN_CYCLES_MBOX,     cycles); //0

MB_PUT8 (RGPIO_DRV_PIN_ACK_MBOX,        ack);//1

MB_PUT8 (RGPIO_DRV_PIN_OVR_MBOX,        ovr); //0

void MB_PUT8(mailbox_t mailbox, uint8_t write_data)

{ _capi_mb_put8(mailbox, write_data);

}

### #ifndef _capi_mb_put8

MB_PUT8 (RGPIO_DRV_PIN_PIN_MBOX,        pin);   //0

这个函数做了如下几件事：

- CPRINT_ARG 地址CAPI 地址所指向的内容中写入数据mailbox（RGPIO_DRV_PIN_PIN_MBOX）RGPIO_TBCOMM_BASE

- CPRINT_ARG 地址CAPI 地址所指向的内容中写入数据write_data（pin）

- CPRINT_CMD地址所CAPI 地址指向的内容中写入数据 command: CPRINT_MB_PUT8 (0x3)  command 定义

#define _capi_mb_put8(mailbox, write_data) \

do \

{ CAST_ASSIGN(CPRINT_ARG, mailbox, mailbox_t); \  实参解析

CAST_ASSIGN(CPRINT_ARG, write_data, uint8_t); \

CAST_ASSIGN(CPRINT_CMD, CPRINT_MB_PUT8, command_t); \  command 定义

NOTIFY_TE_MB_PUT8((mailbox_t)(mailbox), (uint8_t)write_data); \

} while(0)

#endif // _capi_mb_put8

// Note: this macro assumes you provide a non-volatile type so that it can

// leave the RHS of the assignment non-volatile, and that there is a

// corresponding volatile type to use for the LHS.

#define CAST_ASSIGN(address, data, the_type) \

do \

{ CAST((address), v##the_type) = ((the_type)(data)); \

} while(0)

#define CAST(address, the_type) (*((the_type*)(&(address))))

V+类型的目的是给类型前面加上volatile，编译器处理的时候不会优化这个变量

// Convenience macros for type recasting

// C coding note: you can not wrap the_type argument with parenthesis as you

// would for most macro arguments.  Wrapping the type would result in code

// like this: (*(((vuint8_t)*)&(address))), which separates the_type from

// the asterisk *, which transforms the cast into a multiplication.

typedef volatile mailbox_t vmailbox_t;

### CAST_ASSIGN(CPRINT_ARG, mailbox, mailbox_t);

#define CPRINT_CMD          (*((vcommand_t*)((CPRINT_REGS_BASE)+(CPRINT_CMD_OFFSET))))

#define CPRINT_ARG          (*((vuint8_t*)((CPRINT_REGS_BASE)+(CPRINT_ARG_OFFSET))))  CAPI 地址

#define CPRINT_RSP          (*((vuint8_t*)((CPRINT_REGS_BASE)+(CPRINT_RSP_OFFSET))))

本函数传入三个变量：CAPI CPRINT_ARG地址，数据mailbox，参数类型mailbox_t

宏展开就就变成:

*(vmailbox_t)(& CPRINT_ARG) = mailbox_t(mailbox)

1. 最终目的是向地址CPRINT_ARG所指向的memory中写入数据mailbox

2. v+ mailbox_t, 目的是将mailbox_t类型 + volatile。

## WRM32

写入数据，然后

比较写入值跟读取值两者mask对应位的数据是否一直

- 向地址address指向的内存中写入write_data

- 从address中读出对应的内容

- 2获取的值跟mask进行位与操作，write_data跟mask进行位与操作。两者比较，一致则pass，否则fail

#define WRM32(address, write_data, mask)                        capi_wrm32((vuint32_t*)&(address), (uint32_t)(write_data), (uint32_t)(mask))

void capi_wrm32(vuint32_t* address, uint32_t write_data, uint32_t mask)

{ _capi_wrm32(*address, write_data, mask);

}

#define _capi_wrm32(address, write_data, mask)                      _capi_wrmx(32, (address), (write_data), (mask))

#ifndef _capi_wrmx

#define _capi_wrmx(x, address, write_data, mask) \

do \

{ uint_t(x) actual_data; \

VUCAST(x, (address)) = (uint_t(x))(write_data); \

actual_data = VUCAST(x, (address)); \

NOTIFY_TE_WRM##x((address_t)&(address), (uint_t(x))(write_data), actual_data, (uint_t(x))(mask)); \

if((actual_data & (uint_t(x))(mask)) == ((uint_t(x))(write_data) & (uint_t(x))(mask))) \

SUCCESS(); \

else \

{ NOTIFY_TB_CA##x##x##x(CPRINT_ERROR_WRM##x, (address_t)&(address), (uint_t(x))(write_data), actual_data, (uint_t(x))(mask)); \

FAILURE(); \

} \

} while(0)

#endif // _capi_wrmx

## BSET32

设置对应bit的值

- 从address中取内容，大小为32bit

- 取出的内容与mask进行按位与操作

- 2中修改后的值写入address对应内存

#define BSET32(address, mask)                                   capi_bset32((vuint32_t*)&(address), (uint32_t)(mask))

void capi_bset32(vuint32_t* address, uint32_t mask)

{ _capi_bset32(*address, mask);

}

#define _capi_bset32(address, mask)                                 _capi_bsetx(32, (address), (mask))

## WAIT_REM32

规定次数内，读取数据，判断这个数据对应mask位跟expect_data 的mask位数据是否一致

#define WAIT_REM32(address, expect_data, mask, read_attempts)   capi_wait_rem32((vuint32_t*)&(address), (uint32_t)(expect_data), (uint32_t)(mask), (uint16_t)(read_attempts))

void capi_wait_rem32(vuint32_t* address, uint32_t expect_data, uint32_t mask, uint16_t read_attempts)

{ _capi_wait_rem32(*address, expect_data, mask, read_attempts);

}

#define _capi_wait_rem32(address, expect_data, mask, read_attempts) _capi_wait_remx(32, (address), (expect_data), (mask), (read_attempts))

#define _capi_wait_remx(x, address, expect_data, mask, read_attempts) \

do \

{ uint_t(x) actual_data; \

uint16_t count=0; \

uint16_t info_count=CAPI_POLLING_INFO; \

do \

{ actual_data = VUCAST(x, (address)); \

if(!--info_count) \

{ NOTIFY_TB_CA##x##x##x##16(CPRINT_INFO_WAIT_REM##x, (address_t)&(address), actual_data, (uint_t(x))(expect_data), (uint_t(x))(mask), count+1); \

info_count = CAPI_POLLING_INFO; \

} \

} while(((++count < (uint16_t)(read_attempts)) || !(uint16_t)(read_attempts)) && ((actual_data & (uint_t(x))(mask)) != ((uint_t(x))(expect_data) & (uint_t(x))(mask)))); \

NOTIFY_TE_WAIT_REM##x((address_t)&(address), actual_data, (uint_t(x))(expect_data), (uint_t(x))(mask), count); \

if(((actual_data & (uint_t(x))(mask)) == ((uint_t(x))(expect_data) & (uint_t(x))(mask))) && ((count <= (uint16_t)(read_attempts)) || !(uint16_t)(read_attempts))) \

{ NOTIFY_TB_CA##x##x##x##16(CPRINT_INFO_WAIT_REM##x, (address_t)&(address), actual_data, (uint_t(x))(expect_data), (uint_t(x))(mask), count); \

SUCCESS(); \

} \

else \

{ NOTIFY_TB_CA##x##x##x##16(CPRINT_ERROR_WAIT_REM##x, (address_t)&(address), actual_data, (uint_t(x))(expect_data), (uint_t(x))(mask), count); \

FAILURE(); \

} \

} while(0)

# 汇编

https://muzico.gitee.io/niao/15701134073849.html#toc_9

## Ldr

https://blog.csdn.net/u012351051/article/details/125957204

[] 的作用类似C中的*， 例[R1],  就是将R1的值作为一个地址进行操作

## Str

https://www.cnblogs.com/lifexy/p/7171719.html

## 案例

以assign1为例：

- Ldr r1, [pc,#628]    // PC+628这个地址0x1ffe2de0(PC=当前指令地址+2=0x1ffe2b68+2)，对应的值传给R1，R1=0X3001ffe8

- Ldr r3, [pc,#632]    //PC+632这个地址0x1ffe2de4对应的值传给r3，r3=0x3131004c.(这个地址中存储了0xaaaaaaaa)

- Str r3, [R1]     //r3的值传入R1这个地址所指定的memory中，即0x3131004c 写入到地址0x3001ffe8对应的内存中

# 编译原理

https://www.cnblogs.com/33debug/p/6545136.html