目录

Test cases	2

DMA functional cases	2

Different core access	2

System related	3

Peripheral access	3

Signals(need to talk which should be exclude)	3

Computer_dma_soc_glue	3

compute_ipd_hsk_dma0	3

compute_ipd_hsk_dma1	4

compute_ipd_hsk_per0	4

compute_ipd_hsk_per1	4

Sense_dma_soc_glue	4

Sense_ipd_hsk_dma2	4

Sense_ipd_hsk_dma3	4

media_dma_soc_glue	4

Media ipd hsk per0	5

Media ipd hsk per1	5

Media ipd hsk per2	5

Media ipd hsk per3	5

# Test cases

RT700 support 4 DMAs, DMA0/1 in computer domain support 104 peripherals(1-104) request.

DMA2/3 in sense domain support 50 request(1-52, 21/22 reserved).

| DMA instance | Supported peripherals in computer domain | Supported peripherals in sense domain | Supported peripherals in media domain |
| --- | --- | --- | --- |
| DMA0 | 2-27, 35-102 | Support 10 peripherals request. Mic(1),sar_adc(28-29), sd_adc(30-33), acmp(34), lpi2c(103-104) | Support 14 peripherals request. Xspi2(6-7), flexion(37-44),spi14(73-74), spi16 (75-76) |
| DMA1 | 2-27, 35-102 | Support 10 peripherals request. Mic(1),sar_adc(28-29), sd_adc(30-33), acmp(34), lpi2c(103-104) | Support 14 peripherals request. Xspi2(6-7), flexion(37-44),spi14(73-74), spi16 (75-76) |
| DMA2 | 0 | 1, 4-20, 31-52 | Support 10 peripherals request. Xspi2(2-3), flexio(23-30) |
| DMA3 | 0 | 1, 4-20, 31-52 | Support 10 peripherals request. Xspi2(2-3), flexion(23-30) |

## DMA functional cases

| Test case | Description |
| --- | --- |
| Register access | Test if DMA0/1/2/3 register can’t be accessed by different core: CM33, CM33_NS, CM33_M, CM33_M_NS, HIFI1, HIFI4, ZENV. Access non-existed register will cause hard fault |
| Interrupt | Test if DMA0/1/2/3 can generate half done/full done interrupt. Cover different core: CM33, CM33_M, CM33_NS, CM33_M_NS, HIFI1,HIFI4,ZENV |
| Error detection | DMA0/1/2/3 should generate expected error according to the designed setup |
| Channel linking | Check if DMA0/1/2/3 can start transition by a new channel after finish current transition |
| Scat gather | DMA0/1/2/3 can load the configuration data from SRAM to register and start the transition successfully |
| Privileged&Security level | Change DMA privileged&security level, dma works well under different configuration |
| Clock&reset off/on | DMA will not give any response if gate the clock, dma transfer data successfully after clock & reset enabled |
| Software trigger transition | DMA can transfer data with different configuration successfully after setting the start bit |
| Memory/peripheral access | DMA can transfer data from secure ram to secure ram, non-secure ram to non-secure ram, ram to peripheral’s register, peripheral’s register to ram |

## Different core access

| Test case | Description |
| --- | --- |
| Register access | Zenv, hifi1, hifi4, cm33, cm33_ns, cm33_m, cm33_m_ns can access the dma register, if access the non-exist register will response the hardfualt |
| Data transition | Zenv, hifi1,hifi4, cm33, cm33_ns, cm33_m, cm33_m_ns can trigger the dma transfer data successfully |
| Interrupt | Dma interrupt works well for Zenv, hifi1, hifi4, cm33, cm33_ns, cm33_m, cm33_m_ns |

## System related

| Test case | Description |
| --- | --- |
| Low power wakeup | Test if DMA can enter sleep mode and wakeup the core by interrupt successfully |
| Dual core boot with SRPG config3 | Dual core boot, dma2/3 in sense domain enter deep sleep and wakeup successfully |
| Debug mode | DMA register configuration and start transition under debug mode successfully |

## Peripheral access

| Test case | Description |
| --- | --- |
| Peripherals request dma0 | Peripheral 1-104 request dma0 transition |
| Peripherals request dma1 | Peripheral 1-104 request dma1 transition |
| Peripherals request dma2 | Peripheral 1-52 request dma2 transition,21-22 reserved |
| Peripherals request dma3 | Peripheral 1-52 request dma3 transition,21-22 reserved |
| Peripheral request with DMA temp wakeup | Flexcomm0-13， flexio0-7，micfil，sai0-2，lpspi spi14/spi16 |

# Signals(need to talk which should be exclude)

## Computer_dma_soc_glue

| Signals | Description |
| --- | --- |
| compute_ipd_hsk_dma0__ipd_req_dma__104_0 | Sense + media domain的peripheral？ 1， 6-7，28-34 ， 37-44， 73-76， 103-104 35-36， 77-80？ code里面提到 |
| compute_ipd_hsk_per0__ipd_done_per__104_0 | Computer domain中async访问dma的 peripheral 77-80(i3c)， 35-36(ITRC). |
| compute_ipd_hsk_per0__ipd_req_sync__104_0 | Computer domain中async访问dma0的 peripheral 77-80(i3c)？35-36(ITRC) |
| media_ipd_hsk_per0__ipd_req_sync__104_0 | Media domain访问DMA0的peripheral， async访问? 6-7(xspi2)， 37-44(flexio)，73-76(spi14,spi16)， |
| sense_ipd_hsk_per0__ipd_req_sync__104_0 | Sense domain访问DMA0的peripheral, async访问 1(mic)， 28-34(sar_adc, sd_adc,acmp)， 103-104(lpi2c) |
| syscon_cmpt__dma0_enable__127_0 | Computer domain访问DMA0的peripheral的syscon的enablement信号。 35-36(ITRC)没用到？ |

## compute_ipd_hsk_dma0

testbench.top.compute_vdd2_ft_wrap.compute_vdd2.compute_ipd_hsk_dma0

## compute_ipd_hsk_dma1

testbench.top.compute_vdd2_ft_wrap.compute_vdd2.compute_ipd_hsk_dma1

## compute_ipd_hsk_per0

testbench.top.compute_vdd2_ft_wrap.compute_vdd2.compute_ipd_hsk_per0

## compute_ipd_hsk_per1

testbench.top.compute_vdd2_ft_wrap.compute_vdd2.compute_ipd_hsk_per1

## Sense_dma_soc_glue

| Signals | Description |
| --- | --- |
| media_ipd_hsk_per2__ipd_req_sync__52_0 |  |
| sense_ipd_hsk_dma2__ipd_req_dma__52_0[52:0] |  |
| sense_ipd_hsk_per0__ipd_done_per__104_0 |  |

## Sense_ipd_hsk_dma2

## Sense_ipd_hsk_dma3

## media_dma_soc_glue

| Signals | Description |
| --- | --- |
| media_ipd_hsk_per2__ipd_done_per__52_0 |  |
| media_ipd_hsk_per3__ipd_done_per__52_0 |  |
| sense_ipd_hsk_dma2__ipd_done_sync__52_0 |  |
| testbench.top.media_vdd2_ft_wrap.media_vdd2.media_dma_soc_glue.compute_ipd_hsk_dma0__ipd_done_sync__104_0[104:0] |  |
| media_ipd_hsk_per0__ipd_done_per__104_0 |  |
| syscon_cmpt__dma0_enable__127_0 |  |

## Media ipd hsk per0

| Signals | Description |
| --- | --- |
| testbench.top.media_vdd2_ft_wrap.media_vdd2.media_ipd_hsk_per0.ipd_req_sync |  |

## Media ipd hsk per1

## Media ipd hsk per2

## Media ipd hsk per3

## Overall coverage

| Module | Signals(toggled/total) | Coverage rate |
| --- | --- | --- |
| DMA0 | 720/734 | 98.9% |
| DMA1 | 635/648 | 97.99% |
| DMA2 | 496/504 | 98.41% |
| DMA3 | 449/454 | 98.9% |
| DMA0 MEM | 360/366 | 98.36% |
| DMA1 MEM | 360/366 | 98.36% |
| DMA2 MEM | 359/371 | 96.77% |
| DMA3 MEM | 359/371 | 96.77% |

| DMA glue | Signals(toggled/total) | Coverage rate |
| --- | --- | --- |
| Compute VDD2 | 7910/8405 | 94.11% |
| Sense VDD1 | 3898/4120 | 94.61% |
| Media VDD2 | 1734/1750 | 99.09% |

# Questions

- Computer domain, DMA0/1. I3C 77-80. Async访问没有打上Y

- CTIMER 用的是ACK 信号，不是done_lw信号